A-APRS v2.0

by Claudio Parmigiani IZ2FER

Quick reference guide

The purpose of this document is to allow the user to quickly configure A-APRS v2.0. This document describes all the customizations and settings in order to meet your needs.

There will not be modifications to service/core parameters and this document assume that the system is composed only by ArduinoUno and Ethernet Shield, without any other device connected. Any other external device might stop working after the loading of the A-APRS v2.0 sketch.

Informations marked in red are necessary for the configuration of A-APRS-2. Do not proceed further if you do not have all the informations.

- Station Callsign ( i.e. IZ2AAA )

- PASSCODE related to your station callsign, generated for example from:  http://www.ir3ip.net/act/aprs.php ( i.e. 12345 )

- Latitude/Longitude of your QTH ( i.e. 45 degrees 00 minutes 00 hundredths of a minute NORTH, 009 degrees 00 minutes 00 hundredths of a minute EAST )

- The text of the beacon that will be transmitted ( i.e. Claudio – Milano )

- Address (URL) of one or more APRS-IS server ( i.e. italys.aprs2.net , euro.aprs2.net, rotate.aprs2.net )

- Address (URL) of your SMTP mailserver ( i.e. smtp.pluto.com )

- E-mail account from which messages will be sent, this address must belong to the server declared in the previous bullet ( i.e. pippo@pluto.com )

- E-mail account where you want to receive the notifications ( i.e. paperino@pluto.com )

- Username BASE64 encoded of your e-mail account, obtained converting your plain-text username.

( i.e. username pippo once encoded became cGlwcG8= ) You can use this web site for encoding: http://www.opinionatedgeek.com/dotnet/tools/base64encode

- Password BASE64 encoded of your e-mail account, obtained converting yoour plain-text password.

( i.e. pa55w0rd once encoded became cGE1NXcwcmQ= ) You can use this web site for encoding: http://www.opinionatedgeek.com/dotnet/tools/base64encode

- Local Area Network parameters. If you do not know these informations please refer to your ADSL modem/router configuration page:

  - Default DNS ( i.e. 192.168.1.1 )

  - Default Gateway ( i.e. 192.168.1.1 )

  - Subnet Mask ( i.e. 255.255.255.0 )

  - Free IP address that will be assigned to Arduino ( i.e. 192.168.1.177 ) It must be unique.

In the following sections mandatory parameters will be marked in red, flags to enable/disable services will be marked in blue, other parameters will be marked in green.

Open your sketch with Arduino IDE v.1.0.2 o 1.0.3 and identify the beginning of the section:

// ********************* USER VARIABLES, CHANGE TO MEET YOUR NEEDS ********

APRS PART

In this section will be used the callsign IZ2AAA, the passcode 12345, three APRS-IS servers, the geographic location, two symbols to set the icon that will appear on to the maps and the beacon text. If you plan to use less than three APRS-IS servers write them more than once, do not leave empty fields in the array serverName[].

const boolean APRSEnable = true;

const char* serverName[]={ "italys.aprs2.net", "euro.aprs2.net", "rotate.aprs2.net"};

const unsigned int port[] = { 14580,14580,14580};

const char auth[] = "user IZ2AAA pass 12345 vers ArduinoUNO 2.0";

const char btext[] = "IZ2AAA>APRS:=4500.00N/00900.00EIClaudio - Milano";

const unsigned long beaconInterval = 21;

To deactivate this function write false instead of true.

To change the interval between beacon transmissions change number 21 with your preferred. ( it should be greater than 20, minimum suggested value for fixed stations).

The pair of symbols / e I determines the icon that will be shown on to the maps, according to the following tables:

Table selected with first symbol /	    		    Table selected with first symbol \

Once the icon has been choosed, fixing consequently the first symbol ( / or \ ), the second symbol to identify the icon will be determined by the following table:

Examples:

/I 	will set the TCP/IP icon

/> 	will set the car icon

\-	will set the HF station icon

Source: http://wa8lmf.net/aprs/APRS_symbols.htm

The symbols must be written individually in the exact position of the string. In the example above they are marked in purple.

If you plan to use a “local” APRS-IS server, not belonging to xxx.aprs2.net domain, you shall contact the SysOp in order to obtain the retransmission of your beacons on to the RF network.

Without this clearance your beacon and your icon will remain visible only to internet users and will not be propagated through the RF network.

WATCHDOG PART

In this section will be defined the number of the DIGITAL PIN to be tested by the watchdog (DIGITAL PIN 5) and the interval between tests expressed in seconds (1).

const boolean WDEnable = true;

const int WDLine = 5;

unsigned long WDInterval = 1;

To deactivate this function write false instead of true.

To modify the number of seconds between tests modify number 1 with your favourite value.

To modify the DIGITAL PIN to be checked modify number 5 with another available DIGITAL PIN.

PUBLIC IP PART

In this section will be defined the interval between tests of the public IP address.

const boolean IPcheckEnable = true;

const unsigned long IPqueryInterval = 31 ;

To deactivate this function write false instead of true.

To modify the number of minutes between checks modify number 31 with your favourite one. (suggested minimum is 20 not to exceed the maximum allowed number of queries)

THERMOSTAT PART

In this section will be defined: ANALOG PIN connected to the temperature probe, target value (Temp), hysteresis, DIGITAL PIN connected to the fan driver (Fan) and the minutes of interval between checks.

const boolean thermostatEnable = true;

const int Probe = 3;

const int Temp = 500 ;

const int Hysteresis = 50;

const int Fan = 8;

const unsigned long ThermostatInterval = 1 ;

To deactivate this function write false instead of true.

To change the ANALOG PIN connected to the temperature probe change number 3 with another one available.

To change the target temperature modify number 500 with the reading corresponding to the desired temperature once sampled by A/D converter of Arduino.

To change the hysteresis of the thermostat modify number 50 with the desired delta.

To change the DIGITAL PIN that will drive the fan change number 8 with another one.

To modify the minutes between checks change number 1 with your favourite one.

IMPORTANT – DO NOT connect probes or devices providing more than 5Vcc to the Probe pin to avoid damaging Arduino.

DNS PART

In this section will be defined the local DNS server and two other public DNSes, one provided by Google and one provided by OpenDNS.

const byte dnsServers[3][4] = {

{ 192, 168, 1, 1 },

{ 8, 8, 8, 8 },

{ 208, 67, 222, 220 }

};

SMTP PART

In this section will be defined the needed settings to allow e-mail delivery, according to the informations described at the beginning of this document. Port number 25 is usually standard for the majority of SMTP servers.

const unsigned int smtpPort = 25;

const char smtpName[] = "smtp.pluto.com";

const char MailFrom[] = "pippo@pluto.com";

const char MailTo[]= "paperino@pluto.com";

const char UserName64[] = "cGlwcG8=";

const char Password64[] = "cGE1NXcwcmQ=";

IP PART

In this section will be defined the needed settings related to your Local Area Network where Arduino will be connected, according to the informations described at the beginning of this document. MAC address must be unique. If you do not have other Ethernet Shields the proposed MAC address should be suitable.

byte mac[] = {  0x00, 0xAA, 0xBB, 0xCC, 0xDE, 0x02 };                                         byte ip[] = { 192, 168, 1, 177 };

byte dns1[] = { 192, 168, 1, 1 };

byte gateway[] = { 192, 168, 1, 1 };

byte subnet[] = { 255, 255, 255, 0 };

PIN/LED DEFINITION

In this section will be defined the DIGITAL PINs connected to the LEDs.

const int aprsGreenLed = 6;

const int aprsRedLed = 7;

const int heartbeatLed = 9;

Green LED is connected to pin 6, red LED to pin 7, heartbeat LED to pin 9. Change, if necessary, keeping in mind that for a proper (smooth) heartbeat it will be necessary to declare a PWM pin for heartbeat LED.

HTML PAGE COUNTER

In this section will be defined the interval, in hours, to clear the html page counter.

const unsigned long htmlCountRollover = 24 ;

Change number 24 with your favourite one.

MEANING OF THE ARDUINO WEB PAGE COUNTERS

RAM shows how many free bytes of RAM are left.

analog0..6 shows the readings of all analog PINs.

dns0..2  shows how many times the three DNSes (0, 1 e 2) have been queried by both functions aprs() and IPcheck().

dns over shows how many times ALL the DNS queries were not fulfilled, and consequently shows how many times the override funtion was used for the server in use in that moment.

APRS0..2 ok/fail shows how many times a specific APRS-IS server was successfully connected (ok) and how many times it was not reached (fail).

pages today shows how many html pages were served in the last 24 hours (by default), counted from device power on.

pages yest. shows how many times were served in the 24 hours before.

pub IP ok/fail shows how many times the internet site used to check the public IP was reached successfully or not.

in pinX 0/1 shows status of DIGITAL PIN X (i.e. 5), PIN X will be tested by function wd(): 1=HIGH=5V, 0=LOW=0V.

out pinY 0/1 shows the status of DIGITAL PIN Y (i.e. 8) related to the function thermostat(): 1=HIGH=5V, 0=LOW=0V

net tests shows how many DNS queries were made, related to function networktest() only.

net resets shows how many times Arduino’s network services were restarted due to internet/LAN connectivity problem