ArdUPS

by Claudio Parmigiani IZ2FER

Quick reference guide for the TCP/IP configuration

The purpose of this document is to allow the user to quickly configure A-APRS v2.0. This document describes all the customizations and settings in order to meet your needs.

Functionality and meaning related to other sections, such as Battery/Current Sensors/Relais/Power Supply have been already described inside the article.

Informations marked in red are necessary for the configuration of ArdUPS. Do not proceed further if you do not have all the informations.

- Address (URL) of your SMTP mailserver ( i.e. smtp.pluto.com )

- E-mail account from which messages will be sent, this address must belong to the server declared in the previous bullet ( i.e. pippo@pluto.com )

- E-mail account where you want to receive the notifications ( i.e. paperino@pluto.com )

- Username BASE64 encoded of your e-mail account, obtained converting your plain-text username.

( i.e. username pippo once encoded became cGlwcG8= ) You can use this web site for encoding: http://www.opinionatedgeek.com/dotnet/tools/base64encode

- Password BASE64 encoded of your e-mail account, obtained converting yoour plain-text password.

( i.e. pa55w0rd once encoded became cGE1NXcwcmQ= ) You can use this web site for encoding: http://www.opinionatedgeek.com/dotnet/tools/base64encode

- Local Area Network parameters. If you do not know these informations please refer to your ADSL modem/router configuration page:

  - Default DNS ( i.e. 192.168.1.1 )

  - Default Gateway ( i.e. 192.168.1.1 )

  - Subnet Mask ( i.e. 255.255.255.0 )

  - Free IP address that will be assigned to Arduino ( i.e. 192.168.1.177 ) It must be unique.

In the following sections mandatory parameters will be marked in red, flags to enable/disable services will be marked in blue, other parameters will be marked in green.

Open your sketch with Arduino IDE v.1.0.3 o 1.0.4 and identify the beginning of the section:

// ********************* USER VARIABLES, CHANGE TO MEET YOUR NEEDS ********

DNS PART

In this section will be defined the local DNS server and two other public DNSes, one provided by Google and one provided by OpenDNS.

const byte dnsServers[3][4] = {

{ 192, 168, 1, 1 },

{ 8, 8, 8, 8 },

{ 208, 67, 222, 220 }

};

SMTP PART

In this section will be defined the needed settings to allow e-mail delivery, according to the informations described at the beginning of this document. Port number 25 is usually standard for the majority of SMTP servers.

const unsigned int smtpPort = 25;

const char smtpName[] = "smtp.pluto.com";

const char MailFrom[] = "pippo@pluto.com";

const char MailTo[]= "paperino@pluto.com";

const char UserName64[] = "cGlwcG8=";

const char Password64[] = "cGE1NXcwcmQ=";

IP PART

In this section will be defined the needed settings related to your Local Area Network where Arduino will be connected, according to the informations described at the beginning of this document. MAC address must be unique. If you do not have other Ethernet Shields the proposed MAC address should be suitable.

byte mac[] = {  0x00, 0xAA, 0xBB, 0xCC, 0xDE, 0x02 };                                         byte ip[] = { 192, 168, 1, 177 };

byte dns1[] = { 192, 168, 1, 1 };

byte gateway[] = { 192, 168, 1, 1 };

byte subnet[] = { 255, 255, 255, 0 };

PIN/LED DEFINITION

In this section will be defined the DIGITAL PINs connected to the LEDs.

const int RelaisLed = 7 ;

const int OperateLed = 8 ;

const int HeartbeatLed = 9 ;

Relais LED is connected to pin 7, Operate LED to pin 9, heartbeat LED to pin 9. Change, if necessary, keeping in mind that for a proper (smooth) heartbeat it will be necessary to declare a PWM pin for heartbeat LED.

HTML PAGE COUNTER

In this section will be defined the interval, in hours, to clear the html page counter.

const unsigned long htmlCountRollover = 24 ;

Change number 24 with your favourite one.

MEANING OF THE ARDUINO WEB PAGE COUNTERS

RAM shows how many free bytes of RAM are left.

analog0..6 shows the readings of all analog PINs.

Load powered by shows what is currently feeding the load:

AC Main = power comes from AC main

BATTERY = power comes from battery

Bat status shows current battery status:

IDLE = charged and unconnected

DISCHARGING = battery is feeding the loads

*CRITICAL* = battery is exhausted

Charging = battery is charging

Not Chargeable = power supply voltage is not enough to recharge the battery

PWRGood shows if the power supply is capable to fully charge the battery:

1 = power supply voltage is enough

0 = power supply voltage is not enough

Relais LineX shoes the pin number and the status of the relais:

1 = relais activated

0 = relais not activated

V Supply LineY shows the calculated voltage [V] applied to the analog pin connected to the power supply

V Distribution LineZ shows the calculated voltage [V] applied to the analog pin connected to the distribution line

V BAT LineJ shows the calculated voltage [V] applied to the analog pin connected to the battery

I Distr LineK shows the calculated current [A] flowing through the distribution line

I Bat LineW shows the calculated current [A] flowing through the battery. Can be positive or negative.

Battery% shows the calculated residual percentage of battery charge (only during discharge)

Ah Charged so far shows, only when charging, how many Ampere*hour [Ah] have been supplied so far to the battery

Ah Last Completed Charge shows how many Ampere*hour [Ah] were supplied to the battery during last complete charge/recharge cycle

Ah Discharged so far shows, only when discharging, how many Ampere*hour [Ah] have been drained so far from the battery

Ah Last Discharge shows how many Ampere*hour [Ah] were drained from the battery during last discharge event

Ah Current Battery Charge shows the residual charge of the battery. This is be reset to the factory value in the Idle status.

Estimated hours shows, during discharge, the estimated hours of duration of the battery

pages today shows how many html pages were served in the last 24 hours (by default), counted from device power on.

pages yest. shows how many times were served in the 24 hours before.

mail sent shows the absolute counter of e-mail submitted successfully to the SMTP server

mail failed shows the absolute number of failures in submitting e-mails to the SMTP server

Max Ah for next recharge shows the charge safety limit for the next recharge

net tests shows the absolute counter of successful tests of network connectivity

net resets shows how many times Arduino’s network services were restarted due to internet/LAN connectivity problem